Singular Values and Damped Inverse Jacobian Coefficient Tuning Guide

Context: this guide is for tuning the damped inverse Jacobian coefficient lambda used in the quadruped leg velocity mapping.

1. Jacobian Velocity Mapping

For leg control, the Jacobian maps joint velocity to foot Cartesian velocity:

x_dot = J q_dot

where x_dot is foot velocity, q_dot is joint velocity, and J is the leg Jacobian.

The inverse problem is: given a desired foot velocity x_dot, find the joint velocity q_dot.

2. What Singular Values Mean

Singular values describe how strongly the Jacobian maps joint velocity into foot velocity along special motion directions.

- Large singular value: good leverage. Small joint motion produces useful foot motion.

- Small singular value: poor leverage. Large joint motion may produce very little foot motion.

- Very small singular value: near a singularity. A normal inverse can command very large joint velocities.

For a Jacobian J, singular values are related to eigenvalues by:

singular values of J = sqrt(eigenvalues of J^T J)

So J^T J has eigenvalues sigma_1^2, sigma_2^2, sigma_3^2, and the singular values are sigma_1, sigma_2, sigma_3.

3. Why the Normal Inverse Can Explode

The normal inverse divides by singular values. In each singular direction, the inverse gain is approximately:

normal inverse gain = 1 / sigma

If sigma becomes very small, 1 / sigma becomes very large. This creates joint velocity spikes near singular or poor-leverage poses.

4. Damped Least-Squares Inverse

The damped inverse Jacobian used in the code is:

q_dot = (J^T J + lambda^2 I)^-1 J^T x_dot

This is equivalent to solving the optimization problem:

minimize ||J q_dot - x_dot||^2 + lambda^2 ||q_dot||^2

- The first term punishes foot velocity tracking error.

- The second term punishes large joint velocity.

- lambda controls the tradeoff between tracking accuracy and joint velocity safety.

5. Effect of Lambda in Singular-Value Form

With damping, the inverse gain in each singular direction becomes:

damped inverse gain = sigma / (sigma^2 + lambda^2)

When sigma is much larger than lambda, this behaves almost like the normal inverse:

if sigma >> lambda, then sigma / (sigma^2 + lambda^2) approximately equals 1 / sigma

When sigma is small, the lambda^2 term prevents the inverse gain from exploding.

6. Rule of Thumb for Choosing Lambda

Compare lambda to the smallest singular value along the trajectory, sigma_min.

- lambda << sigma_min: weak damping, behaves almost like the normal inverse.

- lambda around sigma_min: strong damping in the weakest direction.

- lambda > sigma_min: very conservative, but foot tracking error increases.

Useful starting rules:

- lambda = 0.01 * sigma_min_normal: very light damping.

- lambda = 0.03 to 0.05 * sigma_min_normal: light damping, often a good starting point.

- lambda = 0.10 * sigma_min_normal: noticeable damping.

- lambda = 0.20 * sigma_min_normal: strong damping.

7. Current Quadruped Value

In the current C++ code, the default is:

lambda = 0.005

The current kinematics use meters, so lambda = 0.005 means about 5 mm in Jacobian length units.

For the tested 500-point trajectory, the approximate smallest singular value range was:

sigma_min approximately 0.150 to 0.174 m

Therefore:

lambda / sigma_min approximately 0.005 / 0.150 = 0.033

This is light damping. It should not distort normal trajectory tracking much, but it gives some protection against numerical spikes.

Conclusion: lambda = 0.005 is reasonable for the current tested trajectory.

8. Practical Tuning Workflow

- Run the target trajectory and compute sigma_min along the path.

- Start with lambda around 0.03 * minimum normal-path sigma_min.

- Check maximum hip, thigh, and shank joint velocities.

- Check foot velocity tracking error: ||J q_dot - x_dot||.

- If joint velocity spikes or commands look noisy, increase lambda.

- If foot tracking becomes too weak or inaccurate, decrease lambda.

- Test near stretched and folded leg poses, not only the nominal walking path.

9. Suggested Values to Try

For the current robot trajectory, reasonable test values are:

lambda = 0.001, 0.005, 0.010, 0.020, 0.050

Expected behavior:

- 0.001: almost pure inverse, least damping.

- 0.005: current value, light damping and reasonable default.

- 0.010: still moderate, more protection.

- 0.020: stronger damping, useful if hardware commands are too sharp.

- 0.050: likely conservative; may introduce visible tracking error.

10. Final Recommendation

Keep lambda = 0.005 for the current verification and normal trajectory tuning.

Increase to 0.010 or 0.020 only if hardware testing shows velocity spikes, chatter, or instability near poor-leverage leg poses.